Communication Protocols in Embedded Systems

Beyond UART, SPI, I2C, Wi-Fi, Bluetooth & Zigbee

Embedded Systems  —  Prof. Abdelsalam Ahmed  |  2026

1. Universal Serial Bus (USB)

USB is one of the most widely deployed communication standards in modern embedded systems, enabling high-speed data transfer and power delivery between a host controller and peripheral devices over a single cable.

- Operates in multiple speed modes: Low Speed (1.5 Mbps), Full Speed (12 Mbps), High Speed (480 Mbps), and SuperSpeed (5 Gbps+).

- Supports up to 127 devices on a single host using a tiered star topology via USB hubs.

- Provides bus-powered operation, supplying up to 500 mA (USB 2.0) or 900 mA (USB 3.0) to connected devices.

- Commonly used to interface microcontrollers with PCs, flash drives, HID devices (keyboards, mice), cameras, and audio interfaces.

- USB CDC (Communications Device Class) allows microcontrollers to appear as virtual serial ports to a host computer.

2. CAN Bus (Controller Area Network)

Originally developed by Bosch for automotive applications, CAN is a robust, multi-master serial communication protocol designed for real-time control in noisy environments without requiring a host computer.

- Uses differential signaling on two wires (CAN_H and CAN_L), providing excellent noise immunity.

- Supports data rates up to 1 Mbps (Classic CAN) and up to 8 Mbps with CAN FD (Flexible Data Rate).

- Each message is identified by an arbitration ID rather than a device address, enabling priority-based bus access.

- Built-in error detection and fault confinement mechanisms make it highly reliable for safety-critical systems.

- Widely used in automotive ECUs, industrial automation (CANopen), medical devices, and robotics.

3. Ethernet

Ethernet is a wired networking standard (IEEE 802.3) used to connect embedded systems to local area networks (LANs) and the internet, offering high-speed, reliable communication over longer distances than typical serial protocols.

- Standard speeds include 10 Mbps, 100 Mbps (Fast Ethernet), 1 Gbps, and higher.

- Uses a MAC address for device identification and the TCP/IP protocol stack for internet connectivity.

- Supports full-duplex communication, allowing simultaneous send and receive operations.

- Commonly integrated into industrial controllers, Raspberry Pi-class SBCs, network appliances, and IoT gateways.

- Power over Ethernet (PoE) variants supply electrical power alongside data through the same cable, simplifying deployment.

4. LoRa Communication

LoRa (Long Range) is a wireless modulation technique developed by Semtech that enables long-range, low-power communication. LoRaWAN is the network protocol layer built on top of LoRa for IoT deployments.

- Achieves transmission ranges of 2–15 km in rural areas and 1–5 km in urban environments.

- Operates in sub-GHz ISM bands (433 MHz, 868 MHz, 915 MHz depending on region), avoiding the crowded 2.4 GHz spectrum.

- Extremely low power consumption, enabling battery-operated sensors to last years without recharging.

- Data rates are intentionally low (0.3–50 kbps), making LoRa suitable for small telemetry payloads, not streaming media.

- Used in smart agriculture, environmental monitoring, asset tracking, and smart city infrastructure.

5. RS-485

RS-485 is a robust serial communication standard that extends the capabilities of RS-232/UART for industrial and long-distance environments. It supports multi-drop networks with multiple devices on a single bus.

- Differential signaling allows cable runs of up to 1,200 meters at low data rates.

- Supports up to 32 driver/receiver units on the same bus, with extended variants supporting hundreds of nodes.

- Half-duplex or full-duplex operation possible; half-duplex is most common in practice.

- Data rates up to 10 Mbps over short distances, decreasing with cable length.

- The foundation of industrial protocols such as Modbus RTU, DMX512 (stage lighting), and PROFIBUS.

Quick Comparison

| Protocol | Type | Range | Speed | Main Use |
| --- | --- | --- | --- | --- |
| USB | Wired | < 5 m | Up to 5 Gbps | PC Peripherals |
| CAN Bus | Wired | < 40 m (1Mbps) | Up to 8 Mbps | Automotive/Industrial |
| Ethernet | Wired | Up to 100 m | Up to 1 Gbps+ | Networking/IoT |
| LoRa | Wireless | 2–15 km | 0.3–50 kbps | Long-range IoT |
| RS-485 | Wired | Up to 1200 m | Up to 10 Mbps | Industrial Control |

Conclusion

Each communication protocol serves a distinct purpose in embedded system design. USB excels at device connectivity with PCs; CAN Bus dominates safety-critical automotive and industrial control; Ethernet provides high-bandwidth network access; LoRa enables ultra-long-range IoT deployments; and RS-485 underpins reliable industrial networking over long cable runs. Choosing the right protocol depends on range, speed, power constraints, and the operating environment.